# Loss-Function and Resonance-Map Methods for Polaritonic Dispersion

## A practical guide to plotting dispersion curves and isofrequency contours without solving complex eigenvalue equations

### Objective

The goal of this task is to construct real-valued spectral maps that reveal where electromagnetic modes lie in momentum-frequency space without first solving the full complex nonlinear eigenvalue problem.

For a lossy dispersive material, an exact mode generally satisfies a source-free equation of the form

Ak,ωu=0,

or equivalently

Dk,ω=0,

where A is a Maxwell or boundary-condition matrix and D is a scalar mode denominator. Because the material functions are complex, the exact roots generally occur at complex ω, complex k, or both.

Instead of solving directly for those complex roots, we will initially restrict the scan coordinates to real values and evaluate a resonance indicator

Pk,ω,

whose maxima occur near the poles or singularities of the underlying electromagnetic problem. A two-dimensional color map of P then provides a practical proxy for the dispersion relation, while a constant-frequency slice provides a proxy for an isofrequency contour (IFC).

This approach is particularly useful for initial code development because it replaces a delicate complex-root problem by grid evaluation, peak finding, and contour extraction.

# 1. Notation and the central idea

Use the same wavevector notation throughout the project.

The full three-dimensional wavevector is

k=kx,ky,kz.

For a surface, interface, or two-dimensional material, define the in-plane wavevector

k∥=kx,ky,

with magnitude

k∥=k∥=kx2+ky2.

Its propagation direction can be parameterized by

k∥=k∥k∥=cosθ,sinθ.

The free-space wavenumber is

k0=ωc.

The exact mode condition can be thought of abstractly as

Dk,ω=0.

For a lossless material, this equation may have roots directly on the real k,ω plane. For a lossy material, the root usually moves away from the real plane. The strategy of this task is therefore:

- choose real values of k and ω;

- evaluate a scalar quantity that becomes large when the system is close to a modal pole or matrix singularity;

- plot this quantity as a color map;

- identify ridges or maxima as approximate modal branches.

The resulting curves are dispersion proxies, not exact complex eigenvalues. In the weak- or moderate-loss regime, their peak positions usually track the real parts of the physical modal dispersions well.

# 2. Four useful resonance indicators

There is no single universal quantity called “the loss function” for every electromagnetic geometry. The appropriate quantity depends on how the mode equation is represented.

The four most useful cases for this project are:

- inverse dielectric loss for bulk longitudinal response;

- inverse effective dielectric denominator for a 2D conducting sheet;

- reflection-coefficient maps for interfaces and slabs;

- smallest-singular-value maps for a general Maxwell or boundary matrix.

These should be implemented as separate but compatible numerical tools.

# 3. Bulk dielectric loss: the simplest case

For an isotropic medium with scalar dielectric function εω, the conventional dielectric loss function is

Lω=-Im1εω.

A strong peak occurs when the denominator is small, typically near a frequency where

Reεω≈0

while the imaginary part remains finite.

This quantity is especially useful for identifying longitudinal material resonances, such as bulk plasmons.

## 3.1 Anisotropic generalization

For a diagonal dielectric tensor

εω=εx000εy000εz,

a convenient direction-dependent longitudinal dielectric function is

εLk,ω=k⋅εω⋅k.

The corresponding directional loss function is

LLk,ω=-Im1εLk,ω.

For example, in the x-y plane,

k=cosθ,sinθ,0,

so

εLθ,ω=εxωcos2θ+εyωsin2θ.

### Important limitation

If the material model is local, so that ε depends on ω but not on k, this bulk loss function contains directional and frequency information but no independent dependence on the magnitude k. It therefore does not by itself generate a dispersing polariton band ωk.

Use this quantity as a diagnostic of material resonances, not as the universal band-structure tool.

# 4. General bulk electromagnetic modes: use the smallest singular value

For source-free propagation in a homogeneous anisotropic medium,

k×k×E+k02εωE=0.

Using

k×k×E=kk⋅E-k2E,

define the Maxwell matrix

Mk,ω=kkT-k2I+k02εω.

An exact mode exists when

Mk,ω

is singular.

For a lossless problem, this is equivalent to

detM=0.

For a lossy problem evaluated on a real k,ω grid, exact singularity is generally lost. Instead, calculate the singular values of M:

s1≥s2≥s3≥0.

Define

smink,ω=s3.

Near a modal solution, smin becomes small.

A convenient dimensionless quantity is

smin=sminsmax.

Then define a plotting proxy such as

PSVDk,ω=-log10smin+δ,

where δ is a small positive numerical floor, for example 10-14.

Large values of PSVD identify nearly singular points.

## Why use a singular value rather than the determinant?

The determinant is mathematically exact in the lossless case, but it is often a poor numerical measure of how close a matrix is to singularity. It depends strongly on the overall scale of all matrix entries and can underflow or overflow. The smallest singular value directly measures the distance to singularity and is usually a more stable quantity for mapping modes. This use of the smallest singular value as a resonance indicator is established in numerical photonics [5].

### First bulk test

Choose a real, constant isotropic dielectric function

ε=εI,  ε>0.

For propagation along x, use

k=kx,0,0.

The exact transverse dispersion is

kx=εωc.

A map of PSVDkx,ω should show a bright ridge exactly on this analytical line in the zero-loss limit.

This should be the first validation of the SVD implementation.

# 5. Isotropic interface: use the reflection coefficient as a surface-mode proxy

The bulk Maxwell matrix does not impose interface boundary conditions and therefore cannot, by itself, identify a surface plasmon polariton.

For a planar interface between two isotropic media with dielectric functions ε1ω and ε2ω, define

kz,j=εjωk02-k∥2,  j=1,2.

Choose the square-root branch corresponding to fields that decay away from the interface for a bound surface mode (note that k∥ value is conserved across planar interfaces, while kz,j needs to be defined in each medium separately).

For TM polarization, the Fresnel reflection coefficient is

rpk∥,ω=ε2kz,1-ε1kz,2ε2kz,1+ε1kz,2.

The surface-plasmon mode occurs at a pole of rp, i.e. when its denominator approaches zero:

Dpk∥,ω=ε2kz,1+ε1kz,2.

Instead of solving

Dp=0

for a complex k∥, scan real k∥,ω and plot either

Prk∥,ω=Imrpk∥,ω,

or, if the sign convention makes the contrast inconvenient,

Pr=Imrp.

An alternative purely numerical indicator is

PDk∥,ω=-log10DpDref+δ.

The bright ridge follows the SPP dispersion. Reflection-coefficient maps are widely used in polariton calculations because poles of the optical response reveal the guided/surface modes; anisotropic-polariton work likewise uses momentum-resolved reflection maps [4,6].

## 5.1 Analytical check

For a single isotropic interface, the familiar SPP dispersion is

k∥ω=ωcε1ωε2ωε1ω+ε2ω.

For weak loss, overlay

Rek∥ω

from this analytical expression on the Pr map. The two should agree closely.

This is the best intermediate exercise before moving to anisotropic sheets or slabs.

## 5.2. An anisotropic half-space: Fresnel coefficient for a diagonal tensor

Why this is a useful next step. The reflection-coefficient approach from Section 5 extends directly to an anisotropic material when the plane of incidence contains a principal crystal axis (or for a uniaxial material with its optical axis normal to the interface). There is no need to search for a complex eigenvalue: evaluate the Fresnel coefficient at real frequency and real in-plane wavevector, then display its magnitude, imaginary part, or denominator. This calculates surface/interface response, not automatically every bulk mode.

Place the interface at z=0. Medium 0 is isotropic, with relative permittivity εdω; medium 1 is a local, nonmagnetic anisotropic half-space, with its principal axes parallel to x,y,z:

ε1ω=diagεxω,εyω,εzω.

Begin with propagation along the x principal axis, so k∥=k∥,0, k∥=k∥, and k0=ω/c. Define the normal wavevector components in the incident dielectric and extraordinary (TM) wave in the anisotropic material:

kz,d=εdk02-k∥2,

kz,e=εxk02-εxεzk∥2.

Use the outgoing-wave or decaying branch of each square root. In the standard p-polarization convention, define convenient (common-factor-free) TM admittances and the reflection coefficient:

Ydp=εdkz,d,  Yep=εxkz,e,

rpk∥,ω=Yep-YdpYep+Ydp=εxkz,d-εdkz,eεxkz,d+εdkz,e.

This is the diagonal-tensor, principal-plane reduction of the anisotropic Fresnel relations given in the supplement of Gold et al. (2025) [8, Supplement, Eqs. S2-S6]. The same result can be obtained by matching tangential electric and magnetic fields. For s polarization, the ordinary-wave component is

kz,o=εyk02-k∥2,  rs=kz,d-kz,okz,d+kz,o.

When all three components become equal to εm, the formula must reduce to the isotropic Fresnel coefficient from Section 5. For propagation along the y principal axis, exchange x and y in the extraordinary-wave formulas, while retaining εz.

The TM surface-mode condition is associated with a small denominator,

Dpk∥,ω=εxkz,d+εdkz,e.

Plotting choices on a real grid:

• Imrp (using one fixed p-polarization convention); cross-check with rp because the sign and residue can vary.

• log101+rp2 to expose bright near-pole features.

• -log10Dp2+δ2, where Dp=Dp/εxkz,d+εdkz,e+δD is a normalized denominator and δ,δD>0 are numerical floors.

A peak or ridge is a surface-mode candidate; it must also satisfy the field-decay/outgoing-wave conditions..

## 5.3. Finite anisotropic film: a denominator map is often the easiest dispersion plot

For a film of thickness d (medium 1) between two semi-infinite media 0 and 2, first consider propagation along a principal axis so that p and s waves are not mixed. Compute kz,j and the single-interface rijp from Section 5.2 for each pair of media. The total film reflection coefficient can be written as

rpfilm=r01p+r12pe2ikz,1d1+r01pr12pe2ikz,1d.

Define the film denominator

Dfilmk∥,ω=1+r01pr12pe2ikz,1d.

Calculate a real-grid map of

PDk∥,ω=-log10​Dfilm2+δ2.

Large PD (small Dfilm) marks candidate slab/surface-polariton branches. Also plot rpfilm or Imrpfilm; comparing them suppresses artifacts from interface-factor singularities and numerical cancellation. Never identify every dark/bright feature as a true bound mode without checking the surrounding light lines and decay conditions.

For many layers, use the reflection recursion described by Gold et al. (2024) [7, Appendix A, Eq. 11] and Gold et al. (2025) [8, Supplement, Eq. S1]. With layer j of thickness dj and normal wavevector kz,j,

Γj-1=rj-1,j+tj-1,jtj,j-1 Γje2ikz,jdj1-rj,j-1Γje2ikz,jdj.

Here ra,b and ta,b are the consistently defined single-interface amplitude coefficients, and Γj is the effective reflection coefficient of the structure below layer j. Initialize at the last interface. A denominator factor is

Dj=1-rj,j-1Γje2ikz,jdj.

For a single film, the two forms agree when the interface sign conventions are used consistently; for reciprocal scalar interfaces r10=-r01. The recursion is particularly convenient because no complex root search is required. Gold et al. (2025) use log10D2 to visualize ENZ/Berreman branches in Fig. 2(b,c) [8], and Gold et al. (2024) show a related denominator map in Appendix B, Fig. 6 [7]. In those figures, low values of log10D2 highlight resonances; PD above reverses the color contrast so resonances appear bright.

## 5.4. Isofrequency maps and the key biaxial limitation

For a slab, repeat the calculation at fixed ω0 over the two in-plane components:

k∥=kx,ky=k∥cosϕ,sinϕ.

The desired map is Pkx,ky;ω0, evaluated directly over a Cartesian grid or from a radial sweep in k∥,ϕ. A single k∥ scan along x is not a full IFC.

For a biaxial tensor diagεx,εy,εz, the exact scalar p-polarized expression of Section 5.2 is valid along the principal axes. At general azimuth ϕ, rotating the tensor normally introduces mixed in-plane components and coupling between s and p polarizations. Therefore one cannot simply replace εx by εxcos2ϕ+εysin2ϕ in the exact retarded Fresnel expression.

There is, however, a useful high-momentum, quasi-electrostatic approximation, where polarization mixing is negligible at leading order. Define

ε∥ϕ,ω=εxcos2ϕ+εysin2ϕ,

κ1≃k∥ε∥εz,  rpES≃εzκ1-εdk∥εzκ1+εdk∥.

Use the decaying or outgoing branch consistent with the physical material. This approximate expression is intended only for k∥≫k0 with a local diagonal permittivity tensor. For a single, local semi-infinite half-space, the magnitude k∥ cancels from rpES: it can identify preferred directions and spectral resonance conditions, but not a full k-dependent surface dispersion. Retain retardation or finite thickness for a useful momentum-dependent IFC map.

For accurate arbitrary-azimuth IFCs, use a 2 x 2 reflection matrix instead of a scalar Fresnel coefficient:

Rk∥,ω=rssrsprpsrpp.

The entries can be calculated from the anisotropic boundary conditions by a 4 x 4 Berreman/scattering-matrix evaluation at each real kx,ky,ω point. This requires solving a small algebraic field-matching problem, but not solving for the complex polariton eigenfrequency or complex in-plane wavevector. For a p-polarized input, plot a proxy such as

Pp=log101+rpp2+rsp2.

This is a general computational extension, not a closed-form result supplied by the four attached papers. For gyrotropic/nonreciprocal WSM tensors, polarization mixing and direction reversal must be handled explicitly; the special Voigt geometry treated in Gold et al. (2024) allows decoupled s and p waves [7, Appendix A]. The current-biased Cd3As2 examples of Gold et al. (2025) additionally use a k-dependent material tensor [8]; such phenomenological nonlocal Fresnel models should be treated cautiously outside their specified regime, because rigorous spatial dispersion can require additional boundary conditions.

A final distinction is essential: reflection-map poles identify interface, film and externally coupled modes. A bulk hyperbolic continuum can appear as a broad region of nonzero Imrp or near-field transmission, without forming a sharp surface pole. Blevins et al. (2025) use momentum-resolved energy transmission (rather than reflection poles alone) to reveal both SPP and hyperbolic-mode contributions in Fig. 2 [9]. Therefore keep the bulk Maxwell IFC/SVD module of Section 4 as a complementary check, not a mandatory first solver.

## 5.5. Recommended minimal assignment and sanity checks

Start with: (i) isotropic Au/air to recover Section 5; (ii) a fictitious uniaxial half-space with real εx=εy≠εz; (iii) one finite uniaxial slab; (iv) a biaxial film scanned along each principal axis; (v) a full azimuthal IFC only after implementing the matrix-reflection route or clearly labeling the high-k electrostatic approximation. For each case, save rp, its numerator and denominator, the selected kz branches, and the plotted proxy.

Check isotropic reduction; invariance under kx↦-kx for an unbiased reciprocal diagonal tensor; agreement of the finite-film formula with the one-layer recursion; convergence under real-grid refinement; and that a semi-infinite local electrostatic result does not falsely yield a dispersing k∥ branch. Compare logD2 and Imrp maps; if they disagree, inspect numerator zeros, branch choices, or polarization conversion. Treat the resulting curves as modal-response proxies, not exact complex eigenvalues.

# 6. Two-dimensional conducting sheet: effective dielectric denominator

For a conducting sheet, the material input is a sheet conductivity rather than a bulk dielectric function.

For an anisotropic sheet with principal axes x and y, write

σω=σxω00σyω.

For propagation along

k∥=cosθ,sinθ,

the projected conductivity is

σeffθ,ω=k∥⋅σω⋅k∥.

For a diagonal tensor,

σeffθ,ω=σxωcos2θ+σyωsin2θ.

In the nonretarded limit, a useful 2D mode denominator is

D2Dk∥,ω=εenv+i k∥2ε0ωσeffθ,ω.

Here εenv is an effective dielectric constant of the surrounding media. For a simple sheet between two isotropic half-spaces, a common electrostatic choice is

εenv=ε1+ε22.

Define the sheet loss function

L2Dk∥,ω=-Im1D2Dk∥,ω.

This is the formulation used in the WTe2 hyperbolic-plasmon analysis, where the calculated loss function is displayed as a pseudocolor dispersion map and its maxima are used to trace the plasmon branches. [1]

The sheet loss map is an especially direct first reproduction target for WTe2. A Fresnel reflection calculation is a complementary check when the sheet boundary condition or a physically justified finite-film dielectric model is supplied; it should not silently replace the published 2D conductivity model.

# 7. Constructing a dispersion map

A dispersion map is a two-dimensional array evaluated on a real grid.

For a fixed propagation angle θ, define

k∥=k∥cosθ,sinθ.

Choose arrays

k∥,1,…,k∥,Nk

and

ω1,…,ωNω.

Evaluate

Lij=L2Dk∥,i,ωj;θ.

Plot Lij as a pseudocolor image with horizontal coordinate k∥ and vertical coordinate ω.

## 7.1 Choosing the momentum grid

Polaritons often occupy a very large range of confinement factors

k∥k0.

It is therefore useful to support both:

- a linear k∥ grid;

- a logarithmic k∥ grid;

- a normalized axis k∥/k0.

Do not assume that a grid appropriate for photons, where k∼k0, is sufficient for strongly confined polaritons, where k∥≫k0.

## 7.2 Display scaling

Loss functions often span several orders of magnitude. Useful plotting choices include

log10L+L0

or percentile-limited color scales.

Do not normalize every frequency slice independently when the goal is to compare physical spectral intensity across frequency. Slice-by-slice normalization is acceptable only as a diagnostic for locating weak branches and should be clearly labeled.

# 8. Extracting dispersion ridges

The simplest definition of a dispersion ridge at fixed k∥ is

ωpeakk∥=arg maxωLk∥,ω.

Alternatively, at fixed frequency,

kpeakω=arg maxk∥Lk∥,ω.

Both are useful, but they are not always identical in a broad or multimode spectrum. The code must state which definition is being used.

The WTe2 paper traces the maximum of the calculated loss function at each given wavevector to obtain the fitted dispersion curves. [1]

## 8.1 Do not use a global maximum blindly

If more than one mode exists, a single slice may contain several local peaks. Use a peak-finding routine and retain all peaks satisfying minimum prominence and intensity criteria.

For each value of k:

- identify all local maxima in Lω;

- reject peaks below a selected prominence threshold;

- assign surviving peaks to branches using continuity from neighboring k values.

A simple branch-tracking cost can be based on

Cmn=ωmki-ωnki+1.

Choose the continuation that minimizes Cmn, subject to a maximum allowed step.

This is much more reliable than storing only the global maximum.

# 9. Widths and damping information

At fixed k, a mode often appears as a finite-width peak in frequency.

If a peak is approximately Lorentzian, measure its full width at half maximum

ΔωFWHM.

An approximate quality factor can then be defined as

Qproxy=ωpeakΔωFWHM.

This is a useful diagnostic but should not be confused with an exact complex eigenfrequency calculation. For strongly non-Lorentzian or overlapping resonances, peak width does not map uniquely to the imaginary part of the true eigenfrequency.

# 10. Constructing isofrequency contours from radial scans

Choose a fixed real frequency ω0.

For each propagation angle θ, calculate the loss function as a function of radial momentum:

Lk∥,ω0;θ.

Then find

kpeakθ;ω0=arg maxk∥Lk∥,ω0;θ.

Convert the result to Cartesian coordinates:

kxθ=kpeakcosθ,

kyθ=kpeaksinθ.

Plot

kxθ,kyθ.

The resulting locus is the loss-function proxy for the IFC.

This is exactly the logic used in the WTe2 analysis: the conductivity is projected for each skew angle, a loss-function dispersion is generated for that angle, and the corresponding wavevector is selected at a specified frequency. [1]

## 10.1 Important rule for hyperbolic IFCs

An open hyperbolic contour may not contain a mode for every angle θ. If no statistically meaningful loss peak is found, record that direction as no mode.

Do not interpolate across missing angular sectors merely to make the IFC look smooth.

## 10.2 Multiple radial branches

A finite slab may support several mode orders. A radial scan can then contain several peaks

kpeak0, kpeak1, …

Each peak should be tracked as a separate IFC branch.

# 11. Constructing IFCs directly on a Cartesian momentum grid

A more general alternative is to avoid the polar parameterization and evaluate the resonance proxy directly on a two-dimensional momentum grid at fixed ω0:

Pkx,ky;ω0.

For example, using the SVD proxy,

PSVDkx,ky;ω0=-log10sminkx,ky;ω0smaxkx,ky;ω0+δ.

The IFC appears directly as a bright ridge in the kx,ky plane.

### Advantages

- no assumption that the IFC is single-valued in polar angle;

- handles several branches simultaneously;

- naturally captures rotated or asymmetric contours;

- well suited to biaxial and off-diagonal dielectric tensors.

### Disadvantage

A dense two-dimensional kx,ky scan is computationally more expensive than a set of radial scans.

For the first WTe2 reproduction, use radial scans. For general TaIrTe4 or low-symmetry materials, eventually implement the Cartesian-grid method as well.

# 12. General anisotropic slabs: when the scalar formula is insufficient

Section 5 gives explicit scalar reflection formulas for principal-axis propagation and finite slabs. For a biaxial material at a generic in-plane angle, compute the full polarization-resolved response instead of inserting an angle-dependent effective permittivity into the exact scalar Fresnel formula.

rpk∥,ω,

The appropriate object is then the 2 x 2 matrix R(k_parallel, omega), with both co- and cross-polarized coefficients.

A 4 x 4 Berreman or scattering-matrix evaluation matches tangential electromagnetic fields at each real frequency and in-plane wavevector. It avoids searching for complex eigenfrequencies but still requires an algebraic field-matching calculation. The simpler scalar coefficient is exact only in the decoupled geometries described in Section 5.2.

For a predominantly TM polariton, an initial proxy is

Prk∥,ω=Imrp.

At fixed angle, plot

Prk∥,ω;θ.

At fixed frequency, plot

Prkx,ky;ω0.

Reflection-map ridges are candidate interface/slab modes, not necessarily bulk hyperbolic eigenbranches. Compare maps of the reflection response with denominator minima and check whether the feature lies inside or outside the relevant light cones. The Nano Letters example [9] uses momentum-resolved energy transmission to reveal high-k hyperbolic continua that are not simply isolated reflection poles.

For the first version, implement the diagonal-tensor principal-axis Fresnel formula and the one-film recursion from Section 5. Upgrade to the 2 x 2 matrix response only for arbitrary-azimuth biaxial or polarization-mixing calculations.

# 13. A general boundary-matrix proxy

For an arbitrary interface or multilayer geometry, the boundary conditions can often be written as

Bk∥,ωa=0,

where a contains the amplitudes of the allowed waves in the surrounding media and layers.

An exact source-free mode requires B to be singular.

Instead of solving

detB=0,

calculate

sminB.

Then define

PB=-log10sminBsmaxB+δ.

This produces a general resonance map without requiring the user to decide in advance which reflection coefficient contains the pole most clearly.

For advanced calculations, compare PB with Imrp. The two should trace the same modal branches when both are implemented correctly.

# 14. Validation sequence: do not start with WTe2

The student should implement the methods in increasing order of complexity.

## Test 1 — Lossless isotropic bulk medium

Input:

ε=4.

Expected analytical dispersion:

k=2ωc.

Required output:

- PSVDk,ω color map;

- analytical dispersion overlaid on the ridge.

The numerical and analytical curves should coincide.

## Test 2 — Add small dielectric loss

Set

ε=4+iε″,

with several small values of ε″.

Expected result:

- the resonance ridge broadens;

- the ridge center remains close to the lossless dispersion for weak loss.

## Test 3 — Isotropic metal/dielectric interface

Use a Drude-Lorentz metal such as Ag or Au against air.

Calculate

Imrpk∥,ω.

Overlay the analytical SPP formula.

Expected result:

- one bright surface-mode branch;

- close agreement with the analytical formula in the weak-loss regime.

## Additional test - anisotropic Fresnel and finite-film reduction

For principal-axis propagation, verify the isotropic limit eps_x = eps_y = eps_z, and compare the one-film formula with the corresponding one-layer reflection recursion. Sweep film thickness while keeping materials fixed; the mode features should change continuously. Then compare k_x and -k_x for a reciprocal diagonal tensor. Only after these checks should the code calculate the azimuth-dependent approximation or full biaxial reflection matrix.

## Test 4 — Isotropic 2D Drude sheet

Use the conductivity convention

σω=iDπω+iΓ.

For Γ=0,

σω=iDπω.

Substitute this into

D2D=εenv+ik∥σ2ε0ω.

The zero-loss condition gives

ω2=D2πε0εenvk∥.

Therefore,

ω∝k∥.

Required output:

- loss-function color map;

- extracted ridge;

- analytical k∥ curve.

All three should agree.

## Test 5 — Artificial anisotropic sheet

Choose simple model conductivities σx and σy.

First set

σx=σy.

The IFC must be circular.

Then choose unequal values with the same sign of the reactive response. The IFC should become closed and anisotropic.

Finally choose a frequency for which

ImσxImσy<0.

The IFC should become open/hyperbolic.

## Test 6 — WTe2

Only after Tests 1–5 pass, substitute the WTe2 conductivity model and numerical parameters.

Required targets:

- reproduce the principal-axis loss maps analogous to Fig. 4a;

- reproduce the imaginary conductivity curves analogous to Fig. 4b;

- extract loss maxima and overlay the ridges;

- calculate IFCs at approximately 250, 340, 380, 460, 480, and 500 cm-1, analogous to Fig. 5b.

Wang et al. use precisely this loss-function strategy and obtain a hyperbolic interval where the two principal imaginary conductivities have opposite signs. [1]

# 15. Practical ridge-extraction algorithm

A robust first implementation can use the following logic.

for ik, k in enumerate(k_grid):
    spectrum = proxy[k_index=ik, :]
    peaks = find_local_peaks(spectrum)
    keep peaks above prominence threshold
    store peak frequency, height, width

track peaks between neighboring k values
using minimum frequency-jump criterion

For IFC extraction:

for theta in theta_grid:
    spectrum = loss(k_grid, omega0, theta)
    peaks = find_local_peaks(spectrum)
    select/track desired mode branch
    k_peak[theta] = selected_peak_position

kx = k_peak * cos(theta)
ky = k_peak * sin(theta)

Store NaN, not zero, if no valid mode is found for a given angle.

# 16. Recommended numerical safeguards

## 16.1 Normalize matrix proxies

For SVD-based maps, use

smin=sminsmax

rather than raw smin whenever the matrix entries vary greatly in scale.

## 16.2 Use adaptive grid refinement

Start with a coarse grid to locate ridges. Then refine only in regions where

P>Pthreshold.

This is much faster than using a uniformly dense grid everywhere.

## 16.3 Keep material interpolation complex-valued

From Task 1, interpolate

Reε,  Imε

or

Reσ,  Imσ

separately, then reconstruct the complex function.

## 16.4 Keep the time convention fixed

All signs in ε, σ, rp, and the loss function depend on the assumed harmonic convention. Use one convention throughout the code, for example

e-iωt.

Store the convention in the material metadata.

## 16.5 Check square-root branches

For interface problems,

kz=εk02-k∥2

has two branches. Select the branch corresponding to outgoing or decaying fields according to the physical boundary condition. Incorrect branch choices can generate convincing-looking but unphysical ridges.

# 17. What can go wrong with real-axis proxy methods?

These methods are intentionally easier than complex root finding, but they are not infallible.

### 1. Strongly damped modes

If the pole lies far from the real axis, the resonance may be too broad to produce a recognizable ridge.

### 2. Material absorption can mimic a mode

A strong material resonance can produce a bright feature even when no confined electromagnetic mode exists. Cross-check with a boundary-condition or SVD proxy when possible.

### 3. Several modes can overlap

A global maximum may jump from one branch to another. Use local peak finding and continuity tracking.

### 4. The bulk Maxwell matrix cannot find a surface mode

Surface and slab modes require the relevant boundary conditions. Use rp, a transfer matrix, or a boundary matrix.

### 5. Very high momentum can invalidate the local material model

At sufficiently large k, spatial dispersion/nonlocality can become important. A local εω or σω model may then cease to be quantitatively reliable.

### 7. A reflection map can miss bulk hyperbolic continua

An extended region of high-k bulk propagation may modify Im(r_p) or near-field transmission without creating a narrow pole. Use the bulk IFC from the Maxwell tensor model or a near-field transmission map as a separate diagnostic; do not infer the absence of bulk hyperbolicity from the absence of a bright reflection ridge.

### 8. Peak position is not the exact complex eigenvalue

The map identifies where the real-axis response is strongest. It does not return the exact complex propagation length or lifetime. Those quantities eventually require either a complex-root calculation or a carefully justified line-shape analysis.

# 18. Suggested Python architecture

Build this as a second layer on top of the material-data package from Task 1.

polaritonic_modes/
|
|-- bulk_maxwell.py
|     M(k, omega, epsilon)
|     svd_proxy(k, omega, epsilon)
|
|-- interfaces.py
|     kz(epsilon, k_parallel, omega)
|     rp_isotropic(...)
|     rp_uniaxial_principal(...)
|     anisotropic_film_rp(...)
|     reflection_denominator_map(...)
|     interface_proxy(...)
|
|-- sheets.py
|     sigma_projected(...)
|     D_2D(...)
|     sheet_loss(...)
|
|-- transfer_matrix.py
|     anisotropic_stack_response(...)
|     reflection_matrix_biaxial(...)
|
|-- scans.py
|     dispersion_map(...)
|     ifc_radial_scan(...)
|     ifc_cartesian_scan(...)
|
|-- peaks.py
|     find_peaks(...)
|     track_branches(...)
|
|-- plotting.py
|     plot_dispersion_map(...)
|     plot_ifc(...)
|
|-- notebooks/
      01_bulk_SVD_test.ipynb
      02_isotropic_SPP_reflection.ipynb
      03_anisotropic_interface_slab.ipynb
      04_2D_Drude_sheet.ipynb
      05_anisotropic_sheet_IFC.ipynb
      06_WTe2_loss_map.ipynb
      07_WTe2_IFCs.ipynb

The material library should remain independent. A mode solver should request only

εω

or

σω

from the Task 1 material object.

# 19. Minimum deliverables for Task 2

| Deliverable | Required calculation | Validation target |
| --- | --- | --- |
| Bulk SVD map | PSVDk,ω | Isotropic analytical light line |
| Interface map | Imrpk∥,ω | Analytical isotropic SPP |
| Anisotropic reflection and slab map | principal-axis r_p, film D map, thickness sweep | isotropic limit; film/recursion agreement |
| 2D Drude map | -Im1/D2D | ω∝k∥ |
| Isotropic-sheet IFC | radial loss scan | circle |
| Artificial anisotropic IFC | radial loss scan | closed ellipse-like contour |
| Artificial hyperbolic IFC | radial loss scan | open contour |
| WTe2 dispersion | principal-axis loss maps | Wang et al. Fig. 4a analogue |
| WTe2 IFCs | angular/radial loss scans | Wang et al. Fig. 5b analogue |
| Branch extraction | local peak finding + tracking | smooth continuous branches |
| Damping test | vary damping parameters | ridge broadening without numerical instability |

Do not proceed to TaIrTe4 or PbTaSe2 until all analytical limiting cases and the WTe2 reproduction work with the same code architecture.

# 20. When to move on to complex eigenvalue solving

The real-axis proxy approach is sufficient for the first stage if:

- the loss or reflection map contains a clear isolated ridge;

- different proxy definitions locate the same branch;

- analytical limiting cases are reproduced;

- the extracted dispersion is insensitive to reasonable grid refinement.

A complex-root solver becomes necessary when the goal changes from locating the branch to accurately determining quantities such as

Imk,  Imω,

propagation length, lifetime, or strongly damped/overlapping modes.

The important computational principle is therefore:

real-grid response map→ridge/IFC extraction

→validated initial dispersion

→complex-root solver only when needed

This staged approach makes the eventual complex-eigenvalue calculation much easier to initialize because the real-axis maps already provide accurate starting guesses for the real parts of the roots.

# References

- C. Wang et al., “Van der Waals thin films of WTe2 for natural hyperbolic plasmonic surfaces,” Nature Communications 11, 1158 (2020). DOI: 10.1038/s41467-020-15001-9.

- A. Nemilentsau, T. Low, and G. W. Hanson, “Anisotropic 2D Materials for Tunable Hyperbolic Plasmonics,” Physical Review Letters 116, 066804 (2016). DOI: 10.1103/PhysRevLett.116.066804.

- G. Álvarez-Pérez et al., “Planar hyperbolic polaritons in 2D van der Waals materials,” Nature Communications 15, 116 (2024). DOI: 10.1038/s41467-023-43992-8.

- G. Hu et al., “Hyperbolic shear polaritons in low-symmetry crystals,” Nature 599, 57–62 (2021). DOI: 10.1038/s41586-021-04328-y.

- S. Li and Y. Y. Lu, “Computing photonic crystal defect modes by Dirichlet-to-Neumann maps,” Optics Express 15, 14454–14466 (2007). DOI: 10.1364/OE.15.014454.

- A. J. Sternbach et al., “Polariton design and modulation via van der Waals/doped semiconductor heterostructures,” Nature Communications 14, 7689 (2023). DOI: 10.1038/s41467-023-43414-9.

7. H. Gold, S. Pajovic, A. Mukherjee, and S. V. Boriskina, "GAGA for nonreciprocal emitters: genetic algorithm gradient ascent optimization of compact magnetophotonic crystals," Nanophotonics 13, 773-792 (2024). Appendix A, Eq. 11; Appendix B, Fig. 6. DOI: 10.1515/nanoph-2023-0598.

8. H. Gold, M. Blevins, A. Mukherjee, S. Pajovic, and S. V. Boriskina, "Dynamically-tunable nonreciprocal epsilon-near-zero photonic platform enabled by current-biased Dirac semimetals," Optical Materials Express 15, 1496-1509 (2025). Fig. 2 and Eqs. 4-5; Supplement Section 1, Eqs. S1-S8. DOI: 10.1364/OME.559903.

9. M. G. Blevins, S. Pajovic, and S. V. Boriskina, "Nonreciprocal Hyperbolic and Surface Modes Enable Tunable Near-Field Heat Transfer in Current-Biased Dirac Semimetals," Nano Letters 25, 8876-8883 (2025). Fig. 2 and associated discussion. DOI: 10.1021/acs.nanolett.5c00655.